Pipeline Report

This report will highlight the main steps involved in the PD pipeline process using MRI data. Through the pipeline process, MRI data will be processed and analyzed in order to segment brain structures related to Parkinson’s disease.

This pipeline can be divided into preprocessing and processing stages, preprocessing includes bias field correction, brain extraction, normalization, etc. Processing these MRI images include registration, segmentation (tissue segmentation and brain parcellation), and labelling. The goal of this pipeline process is to label PD-sensitive regions of all participants’ MRI data, where all of these images must share a common intensity level and are aligned together.

In this report, I will list all the tools, along with their implementation, that are used throughout the pipeline process. This pipeline considers two types of images, structural-based images (T1-w) and diffusion-based images(dwi), where each type is processed separately. Here, I will use subject 3102 to develop the pipeline, which is located at Graham cluster on Compute Canada “home/projects/rrg-maclab/ext-bids/PPMI/bids/sub-3102/ses-SymptomaticTherapy”.

- Pre-processing: The goal is to introduce common intensity images.

- T1w

  - Data conversion: MRI data received from scanners comes in DICOM (.dcm) format. This format needs to be converted into NIFTI format (.nii.gz) using tools like dcm2niix (dcm2niix is a command-line tool). A GUI called MRIcroGL performs the data conversion. MRIcroGL and dcm2niix can also organize the resulting NIFTI format images into BIDS format.

  - Brain extraction: FSL BET tool is used to remove non-brain areas (including skull) from structural images. Fsl is a Neuroimaging analysis tools. The GUI FSL BET can be used in this step, and you can assign the directory for the output file.

Alternatively, we can use synthstrip inside a container (apptainer/singularity). The command to run synthstrip inside apptainer is

singularity run synthstrip_1.7.sif \

-i /home/asma/Desktop/Research/RA_Carleton/pipeline/sub-3102/anat/sub-3102_ses-SymptomaticTherapy_run-1_T1w.nii.gz \

-o /home/asma/Desktop/Research/RA_Carleton/pipeline/synthstrip/brain_extracted.nii.gz \

-m /home/asma/Desktop/Research/RA_Carleton/pipeline/synthstrip/brain_mask.nii.gz

We can also use synthStrip in a container to extract the brain (snakedwi uses this tool for brain extraction). In the current nextflow pipeline (on linux/CC), we use Synthstrip, as it gives a more accurate results.

  - Bias field correction: we use N4 tool to remove bias presented in MRI images in order to have the same intensity values across the same brain tissues. N4 is a feature used in ANTs. We need to install ANTs first. ANTs is an Advanced image registration.

We use the brain extracted image from the previous step (fsl bet). N4 will create a bias corrected version of the brain extracted image. We can use itk snap to visualize the intensity differences of the brain extracted image before and after applying N4.

N4BiasFieldCorrection –d 3 –i sub-3102_ses-SymptomaticTherapy_run-1_T1w_brain.nii.gz -o sub-3102_ses-SymptomaticTherapy_run-1_T1w_brain_corrected.nii.gz

  - Intensity normalization: to maintain all images at a common scale for the sake of comparability across images. We use fslmaths for custom normalization. By applying fslmaths, each voxel within an image is divided by the mean intensity.

fslmaths sub-3102_ses-SymptomaticTherapy_run-1_T1w_brain_corrected -inm 1000 /home/asma/Desktop/Research/RA_Carleton/pipeline/sub-3102/tools/fslmaths/sub-3102_ses-SymptomaticTherapy_run-1_T1w_brain_corrected_normalized

This command will locate the output in a specific directory. If only given the name of output file, the output file will be in the same directory as the input file.

- Dwi: Here, we use some FSL tools dedicated specifically for diffusion MRI, like, eddy, DTIFIT, BEDPOST.

  - Fslroi: to estimate non-diffusion weighted images (b0 image). Using images without any diffusion weighting applied is easy to estimate filed inhomogeneity. The input to fslroi is 4d dwi image in NIFTI format, the output data would be images without any diffusion weighting applied. The output of fslroi is used as the input to topup to only estimate the field inhomogeneity.

fslroi sub-3102_ses-SymptomaticTherapy_run-1_dwi.nii.gz PA 0 1

  - TopUp to estimate the filed inhomogeneity and correct it by applytopup. First, we use topup to estimate filed inhomogeneity using the b0 weighting image extracted from fslroi.

topup --imain=PA.nii.gz --datain=acqparams.txt --config=b02b0.cnf --out=PA_topup

Second, distortions caused by magnetic field inhomogeneities will be corrected by applying topup using applytopup.

applytopup --imain=sub-3102_ses-SymptomaticTherapy_run-1_dwi.nii.gz --inindex=1 --datain=acqparams.txt --topup=PA_topup --method=jac --out=PA_corr

The input to applytopup is the original diffusion image along with the field inhomogeneity extracted from topup. The output of applytopup is Jacobian-modulated, distortion-corrected (caused by magnetic field inhomogeneity) DWI data.

  - Brain extraction: FSL BET is used to extract the brain from dwi images and obtain the brain mask that is going to be used with fsl eddy. It is not recommended to apply FSL BET on dwi images directly, it is better to apply it on a non-diffusion weighted image (b0 weighted image). To do so, we will use fslroi to extract the b0 weighted image from the corrected dwi image (magnetic field distortions).

First, fslroi is applied on the output image from applytopup.

fslroi PA_corr.nii.gz PA_1stVol 0 1

Second,fsl bet is applied on the b0 weighted image (output of fslroi).

bet PA_1stVol.nii.gz PA_brain -m -f 0.2

It is better to extract the brain from a b0 image rather than a diffusion weighted image to reduce diffusion processing in the brain region. This brain mask is necessary for fsl eddy tool.

  - FSL eddy correct tool: After removing field inhomogeneities distortions from dwi data, we now need to remove another type of distortion known as eddy currents. FSL eddy is used to correct distortion caused by current in dwi images. Here, we use the original dwi data for the main input file, acquisitions parameters and index files (.txt files), estimated magnetic field inhomogeneity from topup (output file from topup), and the brain mask extracted from fsl bet. The brain mask is extracted from the b0 weighted image, where the b0 weighted image extracted from the corrected dwi image (output of applytopup).

However, using the brain mask extracted from a T1-weighted structural image in eddy is not recommended due to differences in resolutions, orientations, and distortions.

eddy --imain=sub-3102_ses-SymptomaticTherapy_run-1_dwi.nii.gz --mask=PA_brain_mask.nii.gz --index=index.txt --acqp=acqparams.txt --bvecs=sub-3102_ses-SymptomaticTherapy_run-1_dwi.bvec --bvals=sub-3102_ses-SymptomaticTherapy_run-1_dwi.bval --fwhm=0 --topup=PA_topup --flm=quadratic --out=PA_eddy_unwarped --data_is_shelled

  - Co-registration (dwi-t1w): in this step, we align diffusion or functional MRI to the preprocessed T1-weighted image using different types of transformations, including, rigid, affine, or nonlinear. Co-registration is performed to ensure the space of anatomical and diffusion data are matched. NiftyReg tool is used for registration.

reg_aladin -ref /home/asma/Desktop/Research/RA_Carleton/pipeline/sub-3102/tools/fslmaths/sub-3102_ses-SymptomaticTherapy_run-1_T1w_brain_corrected_normalized.nii.gz -flo PA_eddy_unwarped.nii.gz -res /home/asma/Desktop/Research/RA_Carleton/pipeline/sub-3102/tools/niftyreg/dwi_registered_to_t1.nii.gz -aff dwi2T1_affine.txt

In the previous command, -ref will include the reference file (T1-weighted image), -flo will have the input file needed to be aligned to the reference file (diffusion or functional image), -res will have the output file after registration, and –aff will have a text file demonstrating the type of registration.

  - Fsl dtifit: This step is called tensor fitting. Typically, dtifit is run after applying eddy.

  - dtifit -k /home/asma/Desktop/Research/RA_Carleton/pipeline/sub-3102/tools/FSL_eddy/PA_eddy_unwarped.nii.gz -o /home/asma/Desktop/Research/RA_Carleton/pipeline/sub-3102/tools/FSL_dtifit/PA_eddy_dtifit -m /home/asma/Desktop/Research/RA_Carleton/pipeline/sub-3102/dwi/PA_brain_mask.nii.gz -r sub-3102_ses-SymptomaticTherapy_run-1_dwi.bvec -b sub-3102_ses-SymptomaticTherapy_run-1_dwi.bval

-k includes the input directory, -o includes the output directory, -m includes the brain mask directory, -r includes the .bvec directory, and –b includes the .bval directory.

  - Bayesian estimation of diffusion parameters. We use FSL BEDPOST tool to estimate diffusion parameters using Markov Chain Monte Carlo sampling to construct posterior distributions over the diffusion parameters. BEDPOSTX performs fiber modeling to prepare all the necessary files for probabilistic tractography. It estimates the fiber orientation distribution. The command for running bedpostx using CLI is slightly different than the previous tools. Here, you would only need to locate all the necessary files in one directory and refer to that directory while running the command. The necessary files are: the diffusion-weighted data (data.nii.gz), the brain mask (nodif_brain_mask.nii.gz), bvecs file (bvecs), and bvals file (bvals). Note that bedpostx is a case-sensitive tool, therefore, the files should be named exactly as shown in the brackets.

bedpostx /home/asma/Desktop/Research/RA_Carleton/pipeline/sub-3102/tools/BEDPOSTX

- Processing:

- T1w

  - Registration to a template: We need to align participants’ images to verified standards. These standards are known as atlases, which define the location and the shape of brain structures in a spatial reference. MNI152 atlas, a reference template, provides a spatial framework to compare different subjects’ brains.  The commands for registration used in fsl are flirt (linear registration) and fnirt (non-linear registration). However, we first need to align T1w images to MNI152 space using NiftyReg tools, namely RegAladin and RegF3D to conduct rigid/affine and nonlinear registration, respectively.

- Linear registration (affine/rigid registration): First, we align a brain image to a template space (e.g., MNI space). This is known as global alignment to ensure that the orientation, scale, and position of images are approximately similar. The input here will be the result of the last tool applied within the preprocessing phase for structural images, which is fslmaths. Here, we register MNI152 1mm atlas to the preprocessed structural weighted image for subject 3102.

reg_aladin -ref $FSLDIR/data/atlases/MNI/MNI-maxprob-thr0-1mm.nii.gz -flo /home/asma/Desktop/Research/RA_Carleton/pipeline/sub-3102/tools/fslmaths/sub-3102_ses-SymptomaticTherapy_run-1_T1w_brain_corrected_normalized.nii.gz -res /home/asma/Desktop/Research/RA_Carleton/pipeline/sub-3102/tools/niftyreg/ brain_corrected_normalized_aligned_t1.nii.gz -aff transform_affine.txt

We can use other MNI files with thr25 and thr50. Thr0 means the MNI image considers only regions with a probability larger than 0, hence all regions and this is great for w Whole-brain registration to MNI. Thr25 and thr50 are useful for extracting specific regions. I suggest using thr25 since the goal here is to segment the striatum.  In this step, we transfer T1w to MNI space.

Steps to get templateFlow files installed on Graham:
1- module load python

2- export TEMPLATEFLOW_HOME/scratch/asma96/pipeline_PD/TemplateFlow_tpls
this step will automatically create “TemplateFlow_tpls” folder.

3- pip install --user templateflow

4- python -c "from templateflow import api as tflow; tflow.get('MNI152NLin2009cSym')"

After linear registration, we will register MNI152 template tissue probability map, MNI152 template atlas segmentation, and MNI template brain mask into subject space to ensures proper alignment. First of all, snakedwi is using a different templates than fsl templates. Snakedwi uses MNI152NLin2009cSym template files which are part of the TemplateFlow repository. Check https://github.com/templateflow.

- Nonlinear registration (Deformable b spline): here, we align local deformations of images through applying nonlinear registration.

reg_f3d -ref $FSLDIR/data/atlases/MNI/MNI-maxprob-thr0-1mm.nii.gz -flo /home/asma/Desktop/Research/RA_Carleton/pipeline/sub-3102/tools/niftyreg/brain_corrected_normalized_aligned_t1.nii.gz -res /home/asma/Desktop/Research/RA_Carleton/pipeline/sub-3102/tools/niftyreg/brain_corrected_normalized_nonlinearly_registered_t1.nii.gz -cpp control_points.nii.gz

  - Segmentation: We will assign each subject’s brain regions to atlas’s defined labels. Here, the cortical and subcortical regions are defined according to MNI152 and Harvard-Oxford. This step is achieved in three steps: 1) extracting regions of interest (ROIs) according to atlases, 2) applying the ROI mask to the registered image from the previous step, 3) applying statistics to the segmented (extracted) region. Note that segmenting MRI data according to Harvard-Oxford assumes the input data is in MNI space, therefore, it is important to align your data to MNI space before doing any segmentation with Harvard-Oxford. MNI152 provides labels to subcortical structures within the brain, including the striatum, while Harvard-Oxford FSL atlas is a probabilistic standard, provides labels to the brain’s cortical and subcortical regions. Standard fsl atlases provide anatomical segmentation.

      - Volume extraction, or ROI extraction (the result is mask). We can extract one region or several regions. In case of segmenting the subregions of the striatum (structural based), we extract several regions as follows:

        - fslmaths $FSLDIR/data/atlases/Striatum/striatum-structural-1mm.nii.gz -thr 1 -uthr 1 -bin striatum_mask_1.nii.gz

        - fslmaths $FSLDIR/data/atlases/Striatum/striatum-structural-1mm.nii.gz -thr 2 -uthr 2 -bin striatum_mask_2.nii.gz

        - fslmaths $FSLDIR/data/atlases/Striatum/striatum-structural-1mm.nii.gz -thr 3 -uthr 3 -bin striatum_mask_3.nii.gz

        - fslmaths striatum_mask_1.nii.gz -mul 1 labeled_1.nii.gz

        - fslmaths striatum_mask_2.nii.gz -mul 2 labeled_2.nii.gz

        - fslmaths striatum_mask_3.nii.gz -mul 3 labeled_3.nii.gz

        - fslmaths labeled_1.nii.gz -add labeled_2.nii.gz -add labeled_3.nii.gz -bin striatum_structural_mask123.nii.gz

With fsl, we can perform both anatomical and connectivity-based segmentation for the striatum using the template in “$FSLDIR/data/atlases/Striatum” directory. With connectivity and anatomical templates in this directory, we can segment the striatum based on its anatomy into three subregions; ventral-striatum, putamen, and caudate. While segmenting the striatum based on its connectivity results into three subregions as well; limbic, executive, and sensorimotor.  In addition, the templates in the “$FSLDIR/data/atlases/Striatum” directory are in Harvard-Oxford atlas.

The three segmented regions in the previous command are structure-based segmentation of the striatum (ventral-striatum, putamen, and caudate).

      - Mask application to the registered image (result is a segmented region): here, we map the atlas labels to each participant’s brain.

fslmaths brain_corrected_normalized_nonlinearly_registered_t1.nii.gz -mas striatum_structural_mask123.nii.gz brain_corrected_normalized_nonlinearly_registered_t1_segmentation_striatum.nii.gz

      - Statistical analysis to the segmented region: we can compute the mean, volume, and standard deviation of the segmented region (output of previous step).

Fslstats brain_corrected_normalized_nonlinearly_registered_t1_segmentation_striatum.nii.gz -M -V -S

  - Surface generation: In this step we need to process several segmentations of participants. All previous steps involve one participant (3102), however, in this step we need to repeat the whole preprocessing and processing steps regarding T1w images to generate several segmentations for a sample of participants. The segmented regions of each participant’s brain are averaged across all participants to generate unbiased group-level segmentation template (initial template) using fsl flirt. Then, the large deformation diffeomorphic metric mapping (LDDMM) registration algorithm is applied to register each participant’s segmented region to the unbiased averaged template. The template is then updated by using the LDDMM algorithm through averaging the segmentations across all participants (generate new template) and registering participants to the new template. This algorithm will be iterating between averaging segmentations (create/update a template) and registering all participants to the template. This will result in a group-level 'particular region within brain’ template in MNI space, which is reflecting that region (i.e., striatum) across all participants.

LDDMM helps in determining structural changes in the striatum. All previous steps are applied to participant 3102, we now will move to processing another four participants to generate a striatum template surface. We will apply the structural image pre-processing steps on the T1w images for the four participants, resulting in a brain version of the brain, bias-field corrected, and normalized. Hence, we end up with three new files, namely, 3105_brain_corrected_normalized.nii.gz, 3106_brain_corrected_normalized.nii.gz, 3107_brain_corrected_normalized.nii.gz, and 3108_brain_corrected_normalized.nii.gz. These files are then aligned to MNI using linear and nonlinear registration. We then segmented the striatum region for each of the four participants, as mentioned earlier. The striatum segmentations for the four participants are named as “striatum_3105.nii.gz”, “striatum_3106.nii.gz”, “striatum_3107.nii.gz”, and “striatum_3108.nii.gz”. The surface generation steps are summarized as follows:

      - Generate an initial striatum template by averaging striatum segmentations across all participants (generate unbiased average for surface generation).

AverageImages 3 initial_striatum_template.nii.gz 0 striatum_310*.nii.gz

      - Use LDDMM algorithm to align all participants’ striatum segmentations to the initial template and update the template through averaging participants’ striatum segmentations (refine the striatum template) using ANTs.

antsMultivariateTemplateConstruction2.sh -d 3 -o striatum_template_ -i 4 -g 0.25     -c 2 -k 1 -m CC -n 0 -s MI -z initial_striatum_template.nii.gz  striatum_310*.nii.gz

The resulting file is called “striatum_template_template0.nii.gz”.

      - Canonical surface mesh generation: Generate the striatum template surface mesh by using the refined striatum template from the previous step through 50% isosurface probability.  We need to create a Canonical surface template for striatum using the refined striatum template from the previous step. Create a python code and name it “code.py” and run it with “python 3 code.py”. The python script is saved in the same directory where the striatum template is saved: “Desktop/Research/RA_Carleton/pipeline/participants/surface_generation”. This code generates a surface template “striatum_template_surface.ply“.

Then, we will register the striatum template to each participant’s striatum segmentation using LDDMM algorithm or ANTs registration with affine initialization, as follows:

antsRegistration -d 3 -r [striatum_template_template0.nii.gz, striatum_3105.nii.gz, 1] -m mattes[striatum_template_template0.nii.gz, striatum_3105.nii.gz, 1, 32] -t affine[0.1] -c [1000x500x250,1e-6,10] -s 4x2x1vox -f 6x4x2 -o reg_3105_

This step computes the deformation fields from the template striatum to each participant’s striatum. The output file is “reg_3105_0GenericAffine.mat”. Hence, this step is repeated for all participants. Then, we apply those deformation fields to the surface mesh “striatum_template_surface.ply“ after converting it into .csv file. To convert the “striatum_template_surface.ply” to .csv, we need to create a .py script to extract the vertices from the surface and then create a dataframe from these vertices and convert it to a csv file. The script that performs this conversion is “ply_to_csv.py”. The output file is “striatum_template_surface_points.csv”. Now, we apply the transformations as follows:

antsApplyTransformsToPoints -d 3 -i striatum_template_surface_points.csv -o warped_surface_3105_points.csv -t reg_3105_0GenericAffine.mat

The output file is associated with participant 3105, and it is “warped_surface_3105_points.csv”, which is the template surface deformed into 3105’s anatomy. Note that the template surface is only one file created from the refined striatum template. This template surface is then deformed into each participant’s anatomy (i.e., striatum segmentation).